Outline and Deadlines

- Chapter 1 – Problem Definition and Research Need
Defines the PEGDA-based nanoparticle-filled electrolyte system, establishes electro–mechanical coupling, defines the interphase as a gradient region, identifies the AFM representativeness gap, and states Stage 1 and Stage 2 objectives.
Due: March 20

- Chapter 2 – Theory and Measurement Foundations
Establishes PEGDA network mechanics, nanoparticle inclusion physics, interphase definition, AFM contact mechanics limits, statistical sampling necessity, fracture mechanics rationale, and the modeling context (DIM motivation only).
Due: March 17

- Chapter 3 – Sample Preparation and Fracture System Design
Documents material formulations, Stöber SiNP synthesis and size distribution, UV curing protocol, sample geometry, engineered fracture device, and traceability.
Due: March 13

- Chapter 4 – AFM Acquisition Protocol (Stage 1 Focused)
Defines AFM configuration, probe and contact model selection, Stage 1 survey scan parameters, particle identification criteria, and acquisition stopping logic.
Due: March 10

- Chapter 5 – Statistical Validation Framework (Stage 1)
Defines particle counting workflow, diameter filtering, isolation criteria, isolation probability modeling, and required map count for 95% confidence.
Due: March 8

- Chapter 6 – Stage 1 Results and Feasibility Decision
Presents baseline PEGDA validation, particle count statistics, diameter distributions, isolation frequency, required map calculation, and explicit decision on whether Stage 2 is justified.
Due: March 6

- Chapter 7 – Conclusions
Summarizes engineered systems, validated statistical framework, Stage 1 findings, scope limitations, and clearly states what was and was not demonstrated.
Due: March 20

- Chapter 8 – Future Work and Modeling Pathway
Defines the required next steps following Stage 1 feasibility determination, including conditional Stage 2 high-resolution interphase mapping, extraction of radial modulus gradients with explicit boundary definition, uncertainty propagation requirements, and the experimental inputs necessary for future diffuse interface modeling (DIM). Clearly distinguishes between what this thesis established (statistical foundation and feasibility) and what remains to be completed (gradient quantification, transport coupling, and model parameterization).
Due: March 20

- Complete Thesis (Fully Revised, Formatted, Citation-Locked PDF)
Due to Advisor: March 30
Due to Committee: April 1

CHAPTER 1

Problem Definition and Research Need

1.1 System Context and Motivation

1.1.1 Material System

Sources: Report 6

This section must:

- Define the system explicitly:

  - Crosslinked PEGDA-based (or analogous UV-cured) polymer network.

  - Inorganic nanoparticles (size range explicitly stated).

  - Electrolyte-relevant function.

Include:

- Measured elastic modulus ranges.

- Measured ionic conductivity ranges (if reported).

- Crosslink density or network descriptors if available.

State clearly:

- These materials are designed to balance mechanical integrity and ionic function.

- Nanoparticle addition modifies network structure and transport pathways.

Every property claim must be cited.

No general polymer background.

1.1.2 Electro–Mechanical Coupling

Sources: Report 7

This section must demonstrate experimentally that:

- Mechanical deformation influences structural integrity or electrochemical performance.

- Microstructure influences local transport behavior.

- Nanoparticles modify both mechanical and functional response.

Examples must include:

- Modulus dependence of structural stability.

- Fracture or deformation behavior under operating conditions.

- Reported coupling between interfacial structure and performance.

Close with a precise statement:

Polymer electrolytes behave as coupled electro–mechanical systems whose performance depends on nanoscale structural organization.

No speculation. Cite.

1.1.3 Interphase as a Gradient Region

Sources: Report 8

This section must:

- Define interphase experimentally:

  - AFM modulus mapping

  - Nanoindentation

  - Spectroscopic mapping

State:

- Reported interphase thickness ranges.

- Observed modulus gradients.

- Evidence that gradients are continuous rather than discrete shells.

Explicitly contrast:

- Gradient representation

- Core–shell simplification

End with:

The interphase is not a geometric boundary, but a spatially varying mechanical and chemical gradient surrounding each nanoparticle.

1.1.4 Effective Modulus and Particle Distribution

Sources: Report 9

This section must demonstrate:

- Effective bulk modulus depends on:

  - Interphase thickness

  - Interphase modulus contrast

  - Particle dispersion and interparticle spacing

Include:

- Experimental comparisons

- Modeling comparisons (with vs without interphase)

- Evidence where gradient-aware models improve agreement with measured modulus

End with:

Accurate prediction of effective mechanical response requires quantitative knowledge of interphase properties and particle distribution.

This completes the motivation.

1.2 AFM as a Tool for Interphase Quantification

This transitions from “why” to “how.”

1.2.1 Nanoscale Modulus Mapping

Sources: Report 5

Must include:

- How modulus is extracted (force–distance per pixel).

- Typical lateral resolution.

- Reported interphase thickness ranges from AFM studies.

- Softer vs stiffer interphase observations.

Critically include:

- Most studies lack formal uncertainty quantification.

- Few report statistical convergence or confidence intervals.

1.2.2 Contact Mechanics and Measurement Bias

Sources: Report 1

Must include:

- Hertz vs DMT vs JKR differences.

- Adhesion regime dependence.

- Indentation depth relative to feature size (~40–45% threshold).

- Minimum detectable modulus contrast rarely defined.

This section establishes measurement constraints.

No tutorial tone.

1.3 The Representativeness Problem

This is the central justification for your thesis.

1.3.1 Particle Identification Uncertainty

Sources: Report 2

Must include:

- Lack of standardized criteria for distinguishing:

  - True nanoparticles

  - Fracture artifacts

  - Debris

  - Secondary domains

- Reliance on modulus thresholds and morphology filters.

- Lack of classification accuracy metrics.

Conclude:

Interphase analysis requires a quantitatively defined particle population.

1.3.2 Sampling and Statistical Power

Sources: Report 3

Must include:

- No validated sampling protocol for 100–200 nm particles.

- Number of maps rarely justified.

- Convergence and confidence intervals rarely reported.

- Surface interrogation limitation of AFM.

Conclude:

High-resolution interphase interrogation must be grounded in statistically justified sampling.

1.4 Gap Statement

This section synthesizes everything.

Must state explicitly:

- Polymer electrolytes require microstructure-informed mechanical understanding.

- Interphases are gradient regions that influence bulk modulus.

- AFM can resolve these gradients.

- The literature lacks:

  - Quantitative particle classification frameworks

  - Sampling convergence criteria

  - Defined uncertainty bounds for interphase extraction

This section must be 1–2 tight pages.

No repetition.

1.5 Research Objectives

Define clearly:

Stage 1

- Quantitative particle identification criteria.

- Particle count statistics.

- Isolation probability.

- Empirical estimate of maps required for statistically reliable interrogation.

Stage 2

- High-resolution modulus mapping of statistically justified isolated particles.

- Extraction of radial modulus gradients with explicit boundary definition.

Modeling Implication

- Define what information would be required for future diffuse-interface parameterization.

- Do not claim to build the model.

End with:

This work establishes a statistically defensible framework for AFM-based nanoparticle identification and interphase interrogation in PEGDA-based nanocomposites, providing the experimental foundation required for future diffuse-interface modeling.

1.6 Scope and Limitations

Must clarify:

- Focus is mechanical interphase quantification.

- Ionic transport modeling is not developed here.

- AFM measures surface-accessible regions.

- Contact mechanics assumptions are explicitly stated.

1.7 Chapter Roadmap

This thesis proceeds sequentially from physical foundations to statistical validation of nanoscale measurement feasibility.

Chapter 2 establishes the theoretical framework for PEGDA-based solid composite electrolytes, including polymer network mechanics, mechanical–ionic coupling, nanoparticle inclusion effects, interphase definition, AFM measurement limits, and the statistical necessity of controlled fracture surfaces.

Chapter 3 documents the engineered preparation system, including formulation definitions, Stöber synthesis of silica nanoparticles, UV curing infrastructure, sample geometry design, and the custom fracture device used to expose internal composite regions reproducibly.

Chapter 4 defines the AFM acquisition protocol, distinguishing Stage 1 survey mapping for particle detection and statistical feasibility from conditional Stage 2 high-resolution interrogation.

Chapter 5 formalizes the statistical framework used to quantify particle frequency, isolation probability, required scan counts, and processing-route consistency.

Chapter 6 presents the results of Stage 1 analysis and makes a clear feasibility determination regarding whether sufficient isolated particles exist to justify interphase interrogation.

Chapter 7 summarizes the engineering and statistical contributions of the work and clarifies the scope of demonstrated findings.

Chapter 8 outlines future work, including high-resolution interphase gradient extraction and eventual integration into diffuse interface modeling frameworks, contingent upon validated Stage 1 feasibility.

CHAPTER 2

Theory and Background

Purpose of Chapter 2

Establish the physical, measurement, and modeling foundations that make the thesis workflow necessary.

This chapter must:

- Define the physics of the PEGDA-based solid composite electrolyte system.

- Define interphase rigorously.

- Define what AFM mechanically measures and under what assumptions.

- Establish statistical necessity for heterogeneous fracture surfaces.

- Establish why controlled fracture is required.

- Establish why Diffuse Interface Modeling (DIM) requires experimentally grounded gradients.

This chapter must not describe what was done.
It must justify why it must be done.

2.1 Crosslinked PEGDA Network Mechanics

Purpose of Section

Define the mechanical behavior of the crosslinked PEGDA system under nanoscale indentation.

Must Include

- PEGDA (400 MW) molecular structure.

- Free-radical UV curing via TPO.

- Crosslink density as primary determinant of elastic modulus.

- Difference between crosslinked thermoset and thermoplastic systems.

- Viscoelastic response under indentation loading rate.

Include:

E*=E'+iE''

Explicitly state:

- AFM indentation extracts an apparent modulus dependent on loading rate and dwell time.

- Cure gradients and compositional gradients produce spatial modulus variation.

- Nanoscale modulus variation is physically expected in heterogeneous crosslinked systems.

No method description.

2.2 Mechanical–Ionic Coupling in PEGDA-Based Electrolytes

Purpose of Section

Establish that PEGDA–LiTFSI systems are mechanically and ionically coupled systems.

Must Include

- LiTFSI coordination with ether oxygen groups.

- Segmental-motion-dependent ion transport.

- Conductivity relation:

σ=nqμ

Explicitly state:

- Increasing crosslink density increases stiffness but may reduce mobility.

- Nanoparticles modify both stress fields and local polymer structure.

- Local mechanical gradients may imply local transport variation.

This section establishes why interphase characterization matters to electrolyte function.

2.3 Nanoparticle Inclusion Mechanics

Purpose of Section

Explain how rigid nanoparticles perturb local mechanical fields.

Must Include

- Inclusion in softer matrix (qualitative Eshelby-type behavior).

- Stress redistribution around rigid inclusion.

- Filler size dependence of perturbation zone.

- Cure perturbation near particle surfaces.

Explicitly state:

- 100–200 nm inclusions generate gradients over comparable length scales.

- Bulk modulus is an emergent average of heterogeneous local responses.

- Without resolving local gradients, bulk measurements cannot isolate interphase effects.

No speculation. Physics only.

2.4 Interface vs Interphase

Purpose of Section

Define “interphase” operationally and rigorously.

Must Include

Clear definition:

Interphase is the spatially extended region surrounding a nanoparticle in which measurable mechanical and/or chemical properties differ continuously from the bulk matrix.

Explicitly state:

- It is not a geometric shell.

- It is not assumed uniform.

- It is defined by measurable gradients.

Must explicitly identify required measurable quantities:

- Interphase thickness

- Modulus gradient

- Particle spatial distribution

These are required inputs for continuum models.

2.5 Diffuse Interface Modeling (DIM) Context

Purpose of Section

Explain why experimentally measured gradients are required for modeling.

Must Include

- Phase-field concept.

- Continuous spatial representation of material properties.

- Ginzburg–Landau-type framework (qualitative description only).

- Why discrete layered “multi-core” interphase models fail for graded systems.

Explicitly state:

- DIM requires spatially varying elastic coefficients.

- Literature lacks a validated workflow linking AFM-measured gradients to phase-field parameterization.

- Experimental interphase thicknesses are reported, but statistical certainty and parameter transfer are not established.

Report Integration

- Report 4 → Used here to establish:

  - Lack of AFM-to-phase-field parameterization workflow.

  - Failure of discrete layered interphase models.

  - Need for continuous gradient representation.

No modeling derivations. Only modeling necessity.

2.6 Atomic Force Microscopy: Mechanical Measurement Limits

Purpose of Section

Define what AFM measures and under what assumptions.

Must Include

- Cantilever as linear spring:

F=kcΔz

- Force–distance curve concept.

- Contact mechanics relation (spherical approximation):

F=43E*R1/2δ3/2

Explicitly discuss:

- Hertz vs DMT vs JKR assumptions.

- Adhesion relevance.

- Effect of indentation depth relative to feature size.

- Tip radius relative to 100–200 nm particles.

- Spatial resolution limitations.

Explicitly state:

- Contact model selection influences extracted modulus.

- Indentation depth must be small relative to feature size to avoid geometric averaging.

- Apparent modulus contrast may be masked by improper model selection.

Report Integration

- Report 1 → Used here to support:

  - Contact model selection (Hertz vs DMT vs JKR).

  - Indentation depth relative to feature size.

  - Minimum detectable modulus contrast.

- Report 2 → Used here to support:

  - Criteria for distinguishing true particles from fracture artifacts.

  - Importance of multi-channel validation (topography + modulus).

No operational instructions. Physics and limitations only.

2.7 Statistical Necessity for Heterogeneous AFM Surfaces

Purpose of Section

Justify why multiple scan regions and statistical convergence are required.

Must Include

- AFM samples localized regions only.

- Particle presence in fracture plane is probabilistic.

- Modulus values vary spatially.

- Selection bias risk in isolated high-resolution maps.

Explicitly state:

Without statistical convergence:

- Interphase thickness may be over- or under-estimated.

- High-resolution scans may not represent the bulk material.

- Modeling inputs become non-defensible.

Report Integration

- Report 3 → Used here to support:

  - Sampling protocol considerations.

  - Scan count requirements.

  - Particle-per-map expectations.

- Report 5 → Used here to support:

  - Lack of reported convergence.

  - Absence of statistical certainty in AFM interphase literature.

  - Need for confidence intervals and uncertainty quantification.

No methods. Only justification.

2.8 Fracture Mechanics for Internal Exposure

Purpose of Section

Explain why controlled fracture is required prior to AFM interrogation.

Must Include

- AFM measures surfaces only.

- Cast surfaces exhibit cure-direction artifacts.

- Internal interphase must be exposed.

Discuss:

- Mode I fracture dominance.

- Crack propagation near rigid inclusions.

- Risk of shear-induced smearing.

- Surface roughness influence on indentation extraction.

Explicitly state:

- Uncontrolled fracture can bias interphase observation.

- Fracture design must minimize plastic deformation artifacts.

No device description. That belongs in Chapter 3.

End-of-Chapter Synthesis Requirement

Chapter 2 must conclude with a synthesis paragraph stating:

- PEGDA-based solid composite electrolytes are mechanically and ionically coupled systems.

- Nanoparticles generate graded interphase regions.

- Diffuse interface modeling requires experimentally measured gradients.

- AFM can measure such gradients, but only under correct contact mechanics assumptions and statistical treatment.

- Therefore, a structured fracture + AFM + statistical validation framework is required.

No preview of methods.
No procedural language.
No discussion of implementation.

CHAPTER 3

Sample Preparation and Fracture System Design

Purpose of Chapter 3

Design and document a reproducible sample fabrication and fracture system capable of producing statistically defensible, AFM-suitable internal surfaces of PEGDA-based solid composite electrolytes.

This chapter must:

- Define material compositions precisely.

- Document SiNP synthesis (Stöber process).

- Define UV curing parameters.

- Define geometry selection and constraints.

- Document the fracture device as an engineered system.

- Establish traceability and reproducibility.

- Explicitly connect design decisions to AFM statistical objectives.

This chapter must describe how the system was engineered, not justify why (that was Chapter 2).

3.1 Material System Definitions

Purpose of Section

Define all formulations investigated with numerical precision.

Must Include (Table Required)

For each system:

- PEGDA (MW = 400)

- TPO wt%

- SiNP wt%

- LiTFSI wt% (if applicable)

- Total batch mass per sample

- Mixing protocol

- Degassing method (if used)

Explicitly state:

- Why selected SiNP loading supports sufficient particle frequency for AFM survey scans.

- Why LiTFSI concentration is electrolyte-relevant.

- Whether particles are surface-modified or bare.

No discussion of interphase here. Just system definition.

3.2 Silica Nanoparticle Synthesis

This section documents in-house nanoparticle fabrication.

3.2.1 Stöber Process

Must Include

- TEOS concentration

- Ethanol volume

- Water volume

- Ammonia concentration

- Reaction temperature

- Stirring rate

- Reaction time

- Washing procedure

- Drying protocol

Characterization Required

- Measurement technique (SEM or DLS)

- Mean diameter ± standard deviation

- Histogram of particle size distribution

Explicitly state:

- Target diameter (~200 nm).

- Justification: compatible with AFM lateral resolution and tip radius limitations.

No theoretical explanation. That was Chapter 2.

3.3 UV Cure System and Polymerization Control

Purpose of Section

Define curing infrastructure and parameters.

Must Include

- UV source type

- Wavelength range

- Measured irradiance (mW/cm²)

- Exposure duration

- Distance from emitter to sample

- Mold material and geometry

- Ambient conditions (if controlled)

Explicitly state:

- Cure direction (top-down).

- Acknowledge possible cure gradient.

- Why fracture exposure mitigates surface bias.

No polymer chemistry discussion — already covered.

3.4 Sample Geometry Design

Purpose of Section

Define geometric constraints imposed by AFM and fracture requirements.

Must Include

- Final dimensions (10 mm × 10 mm × 6 mm).

- Rationale for square cross-section.

- Chamfered stress concentrator design.

- CAD drawing (required).

Explicitly state:

- Geometry chosen to enforce predictable crack initiation.

- Must fit AFM stage constraints.

- Must produce internal region suitable for survey scanning.

This is mechanical design reasoning only.

3.5 Fracture System Design

This is a full engineering subsection.

3.5.1 Design Requirements

List explicit measurable requirements:

- Induce crack at predefined stress concentrator.

- Promote Mode I-dominant loading.

- Minimize torsion and shear.

- Avoid plastic smearing at fracture surface.

- Allow one-operator use.

- Produce reproducible crack path.

- Maintain sample alignment.

No narrative. Clear engineering criteria.

3.5.2 Device Architecture

Must include:

- CAD model of fracture jig.

- Loading direction diagram.

- Force transmission mechanism.

- Contact surfaces.

- How compressive force generates tensile opening.

- Assembly photos.

Explicitly state:

- How design reduces asymmetric loading.

- How crack path reproducibility was achieved.

3.5.3 Fracture Surface Validation

Must include:

- Representative fracture surface images.

- Surface roughness metrics (if available).

- Qualitative assessment of particle cross-sections exposed.

- Statement on repeatability.

Explicitly state:

- Whether fracture preferentially debonds particles or cuts through matrix.

- Whether fracture morphology is suitable for AFM indentation.

This section must prove fracture suitability, not assume it.

3.6 Cleaning, Handling, and Traceability

Purpose of Section

Ensure reproducibility and dataset traceability.

Must Include

- Cleaning protocol prior to AFM.

- Solvent used (if any).

- Drying method.

- Sample labeling convention.

- Batch naming structure.

- Orientation marking method.

Explicitly state:

- How samples are tracked from synthesis → fracture → AFM.

3.7 Design Constraints Linked to AFM Statistical Framework

This is critical.

This section must explicitly connect Chapter 3 decisions to Chapter 5 statistical goals.

Must include:

- Expected particle frequency per fracture area.

- Minimum particles required per map (numerical target).

- Target scan size (µm × µm).

- How filler wt% supports sufficient particle density.

- Why fracture geometry must expose statistically representative regions.

Explicitly state:

Sample preparation design is not aesthetic — it is driven by the requirement to obtain statistically defensible particle counts for interphase interrogation.

No statistics derivation here. Just design–statistics linkage.

End-of-Chapter Requirement

Chapter 3 must end with a short synthesis paragraph stating:

- Material formulations were precisely defined.

- SiNP synthesis produced controlled particle size.

- UV curing was standardized and documented.

- Sample geometry and fracture system were engineered for reproducibility.

- The preparation system enables statistically valid AFM survey mapping.

No reference to results.
No discussion of data processing.

CHAPTER 4

AFM Acquisition Protocol for Particle Detection and Statistical Feasibility

Purpose of Chapter 4

Define a reproducible AFM acquisition protocol designed to:

- Detect SiNPs on fracture surfaces.

- Quantify particle frequency per scan area.

- Confirm particle identity via modulus contrast.

- Acquire sufficient data to evaluate statistical feasibility for interphase interrogation.

This chapter defines how data are acquired.
It does not define how data are processed (Chapter 5).
It does not quantify interphase gradients.

4.1 Instrument Configuration

Must Include

- AFM model and manufacturer.

- Operating mode (PinPoint / PF-QNM / force–distance mapping).

- Cantilever type.

- Nominal spring constant.

- Nominal tip radius.

- Calibration method for spring constant.

- Deflection sensitivity calibration procedure.

- Ambient conditions.

Explicitly state:

- Force–distance curves are collected at each pixel.

- Modulus values are extracted per pixel using the selected contact model.

- Calibration is performed prior to acquisition sessions.

No contact mechanics derivation (covered in Ch 2).

4.2 Probe and Contact Model Selection

Purpose

Define how modulus contrast is obtained for particle validation.

Must Include

- Tip geometry.

- Nominal radius.

- Selected contact model (Hertz / DMT).

- Justification for model choice for PEGDA-based thermoset.

Explicitly state:

In Stage 1, modulus contrast is used solely to distinguish silica particles from polymer matrix and fracture artifacts. Interphase gradients are not extracted in this stage.

That sentence must exist.

4.3 Stage 1 – Survey Mapping Protocol (Particle Detection)

This is the core of the chapter.

Purpose

Acquire statistically meaningful survey maps for particle detection and counting.

Must Define

- Scan size (µm × µm).

- Pixel resolution (e.g., 512 × 512).

- Lateral resolution (nm/pixel).

- Indentation setpoint force.

- Maximum indentation depth.

- Scan rate.

Must State

- Survey maps prioritize area coverage over maximum resolution.

- Scan size is selected to capture multiple particles per field.

- Multiple spatially separated regions per sample are scanned.

Must Include

- Minimum number of maps targeted per formulation (initial target).

- Sampling distribution strategy across fracture surface (randomized or grid-based).

- Whether forward/backward scans are retained.

No particle isolation yet.
No interphase gradients.
No modeling language.

4.4 Particle Identification During Acquisition

Purpose

Define real-time criteria used to identify candidate particles.

Must Include

Particles must satisfy:

- Diameter within defined bounds (numeric threshold).

- Modulus contrast consistent with silica.

- Geometric shape consistent with spherical inclusion.

- Not fracture debris or pull-out void.

Explicitly state:

- Topography and modulus channels are used jointly.

- Ambiguous features are excluded at acquisition stage.

Isolation is not yet enforced here — only detection.

4.5 Data Volume Targeting

Purpose

Define acquisition stopping logic for Stage 1.

Must Include

- Initial scan count per formulation.

- Condition to continue scanning (insufficient particle frequency).

- Condition to stop (sufficient data for statistical evaluation).

Explicitly state:

Acquisition continues until sufficient data exist to compute particle frequency statistics and isolation probability (defined in Chapter 5).

4.6 Stage 2 – High-Resolution Interphase Scans (Conditional)

This section must be short.

Must State

- Stage 2 scans are only performed after Stage 1 statistical feasibility is established.

- Stage 2 uses reduced scan size and increased resolution.

- Stage 2 targets isolated particles only.

No further detail.
Chapter 5 determines whether Stage 2 is justified.

End of Chapter 4

Conclude with:

- Stage 1 acquisition focuses on particle presence and frequency.

- Modulus is used for material validation only.

- Data volume is driven by statistical requirements.

- Interphase interrogation is conditional on Stage 1 results.

CHAPTER 5

Statistical Validation Framework for Particle Presence and Feasibility

Purpose of Chapter 5

Define the statistical framework used to determine whether fracture surfaces contain a sufficient number of isolated SiNPs to justify high-resolution interphase interrogation.

This chapter answers one question:

Are there enough statistically representative, isolated particles to proceed to Stage 2?

This chapter does not extract interphase gradients.

5.1 Data Organization and Preprocessing

Must Include

- Channels imported.

- Plane leveling procedure.

- Row correction.

- Masking workflow in Gwyddion.

- Export format.

- Python aggregation workflow.

Explicitly state:

- Raw data are preserved.

- Masking thresholds are configuration-defined.

- All processing steps are reproducible.

No statistical interpretation yet.

5.2 Particle Detection and Counting

This is the core of Stage 1.

Must Include

For each formulation:

- Total number of maps analyzed.

- Particle count per map (table format).

- Total particle count across maps.

- Mean particles per map.

- Standard deviation.

- Minimum per map.

- Maximum per map.

- Histogram of particle count distribution.

Explicitly state:

- Particle occurrence treated as probabilistic event.

- Particle detection requires both geometric and modulus validation.

No isolation yet.

5.3 Particle Diameter Distribution

Must Include

- Raw diameter values for all detected particles.

- Mean diameter.

- Standard deviation.

- Histogram.

- Numeric filtering bounds used.

Explicitly state:

- Diameter distribution confirms synthesis consistency.

- Extreme outliers removed only under defined criteria.

No interphase thickness discussion.

5.4 Isolation Analysis

Now we move to feasibility.

Must Include

- Numeric definition of isolation (center-to-center distance threshold).

- Number of isolated particles per map.

- Percentage of maps containing ≥1 isolated particle.

- Distribution of isolation counts.

Explicitly state:

- Only isolated particles are eligible for Stage 2 interrogation.

5.5 Isolation Probability Model

Formal probability section.

Define:

Let p= probability a map contains ≥1 isolated particle.

P(at least one suitable map in N)=1-(1-p)N

Solve for N such that:

P≥0.95

Explicitly state:

- Required number of maps for 95% confidence.

- Whether current dataset meets requirement.

This is the statistical heart of Stage 1.

5.6 Processing Route Validation (Secondary)

This section ensures pipeline credibility.

Must Include

- Comparison of Gwyddion vs Python statistics.

- Effect of outlier filters.

- Δμ and ΔN definitions.

- Forward–Backward scan comparison.

Explicitly state:

- Baseline averages sensitive to extreme values.

- Median may better represent central tendency.

- Processing differences quantified and bounded.

Keep this secondary to particle statistics.

5.7 Stage 1 Decision Statement

This section must end with a single clear statement:

For each material system:

Based on the current dataset, X maps are required to achieve 95% confidence of obtaining an isolated particle. The current dataset includes Y maps. Therefore, Stage 2 is (justified / not justified).

No ambiguity.
No interpretation beyond feasibility.

CHAPTER 6

Results and Discussion

Purpose of Chapter 6

Present and interpret:

- Processing validation results (baseline modulus consistency).

- Stage 1 particle detection results.

- Isolation feasibility analysis.

- Decision on whether Stage 2 is justified.

This chapter must not:

- Quantify interphase gradients.

- Claim mechanistic interphase thickness.

- Make modeling claims.

- Over-interpret modulus variation.

This chapter answers:

Are particles present in statistically sufficient quantity and isolation to justify interphase interrogation?

That is the lead.

6.1 Processing Validation – Baseline PEGDA (No SiNP)

Start here.

Why?

Because the pipeline must be credible before particle claims are made.

6.1.1 Baseline Modulus Consistency

Material:
PEGDA + 1 wt% TPO (no SiNP)

Must present:

- Number of maps analyzed.

- Mean modulus per map.

- Standard deviation per map.

- Histogram of map-level means.

- Forward vs backward comparison.

Explicitly interpret:

- Variation is random around a central tendency.

- No spatial clustering.

- No directional bias.

- Processing route differences bounded and quantified.

Key statement required:

The baseline PEGDA dataset demonstrates that the acquisition and processing pipeline produces internally consistent modulus statistics across spatially distributed scans.

This establishes trust.

No particle talk yet.

6.2 Stage 1 – Particle Presence in PEGDA–SiNP

Now move to particle-containing systems.

6.2.1 Map Inventory

For PEGDA–SiNP:

- Total number of maps.

- Scan size.

- Pixel resolution.

- Total scanned area.

Must include a summary table.

No interpretation yet.

6.2.2 Particle Count Per Map

Present:

- Table: particle count per map.

- Total particle count.

- Mean particles per map.

- Standard deviation.

- Minimum.

- Maximum.

- Histogram.

Explicit interpretation:

- Particle presence confirmed.

- Distribution across maps is (uniform / variable).

- No maps contain zero particles (if true — if not, state explicitly).

No interphase language.

6.2.3 Particle Diameter Distribution

Present:

- All detected particle diameters.

- Mean ± SD.

- Histogram.

- Comparison to synthesis target (~200 nm).

Explicitly interpret:

- Diameter distribution consistent with Stöber synthesis.

- Filtering criteria did not artificially narrow distribution.

Still Stage 1 only.

6.3 Isolation Analysis

Now we move to feasibility.

6.3.1 Isolation Count Per Map

Present:

- Isolation threshold (numeric).

- Number of isolated particles per map.

- Percentage of maps with ≥1 isolated particle.

Explicit interpretation:

- Isolated particles occur in X% of maps.

- Isolation frequency sufficient / insufficient.

6.3.2 Required Map Count for 95% Confidence

Present:

- Computed probability p.

- Required N maps for 95% confidence.

- Actual maps collected.

Explicit decision statement required:

For PEGDA–SiNP, the current dataset (Y maps) exceeds / does not exceed the required N maps for 95% confidence of obtaining at least one isolated particle.

No ambiguity.

6.4 PEGDA–LiTFSI–SiNP Comparison (If Applicable)

Repeat same structure:

- Map inventory.

- Particle count per map.

- Diameter distribution.

- Isolation frequency.

- Required N.

Explicitly compare:

- Does salt change particle detectability?

- Does salt change apparent particle density?

- Does salt increase fracture debris ambiguity?

No transport speculation.
No modulus gradient discussion.

6.5 Processing Route Sensitivity

Now bring back the pipeline comparison.

Present:

- Δμ across processing methods.

- ΔN across filtering strategies.

- Forward–Backward ratio consistency.

Explicit interpretation:

- Processing route affects mean modulus by X%.

- Outlier filtering reduces N by Y%.

- Particle count unaffected / minimally affected.

State clearly:

Particle detection and isolation statistics are robust to reasonable processing variations.

That is the purpose of this section.

6.6 Stage 1 Decision

This section must be explicit and short.

For each formulation:

- Are particles present?

- Is diameter distribution acceptable?

- Is isolation frequency sufficient?

- Is required map count met?

End with:

Stage 1 statistical feasibility has been established / not established for [material system]. Therefore, Stage 2 high-resolution interphase interrogation is justified / not justified.

No hedging.
No future work tone.
Clear decision.

6.7 Discussion – Measurement Implications (Controlled)

Now you are allowed limited interpretation.

Must focus on:

- Fracture suitability.

- Particle detectability.

- Statistical convergence behavior.

- Implications for future interphase interrogation.

Must NOT:

- Report interphase thickness.

- Claim modulus gradients.

- Discuss DIM parameterization in detail.

This discussion should read as:

The engineered preparation and acquisition framework successfully (or unsuccessfully) produces statistically sufficient particle exposure for nanoscale interrogation.

That’s it.

End of Chapter 6

Conclude with:

- Pipeline validated on baseline PEGDA.

- Particle presence confirmed in SiNP systems.

- Isolation probability quantified.

- Required scan count determined.

- Stage 2 decision made.

No overreach.

CHAPTER 7

Conclusions

Purpose of Chapter 7

Summarize the validated contributions of this work.

This chapter must:

- Re-state the problem.

- Summarize what was built and validated.

- State clearly what was demonstrated.

- State clearly what was not demonstrated.

- Make a defensible Stage 1 decision statement.

No speculation.
No overreach.
No modeling claims beyond what was proven.

7.1 Problem Restatement

Open with one paragraph that clearly states:

- PEGDA-based solid composite electrolytes are mechanically and ionically coupled systems.

- Nanoparticles introduce interphase regions that require nanoscale interrogation.

- AFM has been used in related systems, but statistical rigor and reproducibility are limited.

- Before interphase gradients can be claimed, particle presence and statistical feasibility must be established.

Then state:

The objective of this thesis was to design and validate a reproducible fracture + AFM + statistical framework capable of determining whether nanoscale interphase interrogation is statistically defensible.

That is the scope.

7.2 Engineering Contributions

This section summarizes what was engineered and validated.

Must include:

1. Material Preparation System

- Controlled PEGDA-based formulations.

- In-house Stöber SiNP synthesis.

- Documented UV curing protocol.

2. Fracture Exposure System

- Designed sample geometry.

- Designed fracture device.

- Repeatable internal surface exposure.

3. AFM Acquisition Framework

- Structured Stage 1 survey mapping.

- Controlled indentation parameters.

- Reproducible acquisition protocol.

4. Statistical Processing Framework

- Configuration-controlled masking.

- Processing route validation.

- Particle counting methodology.

- Isolation probability modeling.

- Required map count determination.

Each bullet should be concrete and measurable.

7.3 Stage 1 Findings

This is the core conclusion.

For each material system, state clearly:

- Particle presence confirmed / not confirmed.

- Mean particles per map.

- Isolation frequency.

- Required number of maps for 95% confidence.

- Whether current dataset meets that requirement.

Must include an explicit decision:

For PEGDA–SiNP (and PEGDA–LiTFSI–SiNP if applicable), Stage 1 statistical feasibility has been established / not established.

No ambiguity.

7.4 What Was Demonstrated

This section must be precise.

You demonstrated:

- Fracture surfaces contain detectable SiNPs.

- Particle frequency can be quantified.

- Isolation probability can be modeled.

- Required scan count can be computed.

- Processing route effects are bounded and quantified.

- Statistical convergence can be evaluated.

You did NOT demonstrate:

- Definitive interphase thickness values.

- Definitive modulus gradient profiles.

- Direct phase-field parameterization.

- Direct correlation between modulus gradient and ionic conductivity.

That clarity increases credibility.

7.5 Implications for Stage 2 and Beyond

This section must be restrained.

State:

- Stage 2 high-resolution interphase interrogation is justified if Stage 1 passed.

- The framework now enables gradient extraction with statistical backing.

- Future modeling efforts (including DIM) require Stage 2 data.

Do not claim:

- DIM parameterization has been achieved.

- Transport–mechanical coupling quantified.

Only say:

The present work establishes the measurement and statistical foundation required for those future investigations.

Final Paragraph Requirement

End with a single clear synthesis paragraph:

- A reproducible fracture–AFM–statistical framework was engineered and validated.

- Particle presence and isolation frequency were quantified.

- Required scan counts were determined mathematically.

- Stage 1 feasibility decision was made.

- The thesis establishes a defensible foundation for nanoscale interphase interrogation in PEGDA-based solid composite electrolytes.

CHAPTER 8

Future Work and Modeling Pathway

8.1 Conditional Stage 2 – High-Resolution Interphase Interrogation

8.1.1 Isolation-Qualified Particle Selection

Define the criteria under which isolated particles identified in Stage 1 become eligible for high-resolution interrogation.
Must include:

- Isolation threshold (center-to-center distance)

- Minimum particle diameter bounds

- Surface quality requirements

- Exclusion criteria (fracture debris, voids, ambiguous modulus contrast)

8.1.2 High-Resolution Scan Parameters

Define acquisition parameters required for gradient extraction:

- Reduced scan size (µm × µm)

- Increased pixel density

- Indentation depth relative to particle radius

- Contact model consistency

- Tip radius constraints

Explicitly state:
High-resolution mapping must preserve indentation depth sufficiently small relative to particle diameter to avoid geometric averaging.

8.1.3 Radial Modulus Gradient Extraction

Define how interphase gradients would be extracted:

- Radial averaging from particle center

- Boundary definition criteria (matrix baseline convergence)

- Smoothing and filtering limitations

- Gradient continuity assessment

No gradient results reported here — only methodology.

8.2 Uncertainty Propagation Requirements

8.2.1 Measurement Uncertainty

Define:

- Contact model sensitivity

- Tip radius uncertainty

- Indentation depth variability

- Pixel-to-pixel noise

8.2.2 Statistical Uncertainty

Define:

- Particle-to-particle variability

- Required number of isolated particles

- Confidence interval requirements for gradient parameters

Explicitly state:
Interphase thickness and modulus contrast must be reported with statistical bounds prior to model transfer.

8.3 Diffuse Interface Modeling (DIM) Integration Pathway

8.3.1 Required Experimental Inputs

Define what DIM would require:

- Interphase thickness (with confidence bounds)

- Continuous modulus gradient profile

- Particle spatial distribution statistics

- Volume fraction consistency

8.3.2 Parameter Mapping Framework

Describe conceptually:

- Spatially varying elastic coefficients

- Gradient representation vs. discrete shell models

- Why multi-core layered representations are insufficient

No modeling derivation. Only experimental-to-model linkage.

8.4 Transport–Mechanical Coupling Extension

Define future work required to connect:

- Local modulus gradients

- Segmental mobility changes

- Ionic conductivity variation

Explicitly state:
Transport modeling is not developed in this thesis and requires additional experimental characterization.

8.5 Expanded Statistical Validation

Future work must include:

- Larger fracture surface sampling

- Cross-batch reproducibility

- Cross-instrument validation

- Alternative probe geometries

Chapter 8 Summary

This chapter defines the structured transition from statistically validated particle detection (Stage 1) to conditional high-resolution interphase gradient extraction (Stage 2), outlines the uncertainty bounds required for defensible gradient quantification, and identifies the experimentally grounded inputs necessary for future diffuse interface modeling of PEGDA-based solid composite electrolytes.